AOE 4065: Air Vehicle Design

Homework 2: Constraint Analysis

Due on or before 25 September 2025 (11:59 PM)

Instructions

- Your submission should be clear, concise, and complete. In preparing it, organize it logically in such a way that it is easy to comprehend and it contains sufficient detail within the specified constraints.

- Clearly state all relevant assumptions, and cite any reference material you use.

- Submit the homework to Canvas. Please use the following convention for naming the file: AOE4065_HW2_Last-name_First-name

Purpose

- To assess your ability to create a constraint analysis plot based on point performance requirements and obtain the design point.

Scenario:

A fighter jet is to be designed with the following performance requirements:

- Maximum Mach of 1.6 at 36,000 ft

- Cruise at 0.87 Mach at 36,000 ft

- Maximum altitude of 50,000 ft maintaining cruise speed

- Combat turns

  - 4.5g turn at 20,000 ft and cruise speed

  - 1.4g turn at 36,000 ft and 1.4 Mach

- Specific excess power of 500 ft/s at 10,000 ft and cruise speed

  - Hint: Read about specific excess power from this resource.

- Takeoff and land at seal-level in less than 4000 ft

  - kslanding=Vlanding/Vstall and kstakeoff=Vtakeoff/Vstall

  - Hint: Read about takeoff and landing from this resource.

Aerodynamic and propulsive characteristics are given to you:

- Maximum CL

  - Takeoff: 1.275665

  - Landing: 1.425909

- Engine Thrust

  - Minimum: 15,000 lb

  - Maximum (with afterburners): 23,770 lb

Lapse rate, α=αdryTmin+AB%αABTmax-αdryTminTmax

| Constraint | WWTO | CD0 | K1 | K2 | Dry thrust lapse, αdry | Afterburner thrust lapse, αAB |
| --- | --- | --- | --- | --- | --- | --- |
| Max Mach | 0.899666 | 0.039317 | 0.213727 | 0 | 0.298293 | 0.57698 |
| Cruise | 0.899666 | 0.016996 | 0.116031 | -0.0063 | 0.27111 | 0.332642 |
| Max Alt | 0.899666 | 0.016996 | 0.116031 | -0.0063 | 0.138789 | 0.17029 |
| Combat Subsonic | 0.899666 | 0.016996 | 0.116031 | -0.0063 | 0.555662 | 0.681777 |
| Combat Supersonic | 0.899666 | 0.040614 | 0.219406 | -0.001 | 0.357862 | 0.556558 |
| Excess Power | 0.899666 | 0.016996 | 0.116031 | -0.0063 | 0.702727 | 0.85355 |
| Takeoff | 1 | 0.051996 | 0.116031 | -0.0063 | 0.939778 | 0.955053 |
| Landing | 1 | 0.061996 | 0.116031 | -0.0063 | 0.939778 | 0.955053 |

Landing

WTOS=sLρgμCLmax, landing+0.83CD0kslanding2β

μ=0.5

Takeoff

TSLWTO=kstakeoff2β2αρCLmax, takeoffgsTOWTOS+0.7CD0βCLmax, takeoff+μ

μ=0.03

Deliverables:

One PDF file with the following information:

- For each requirement, report the following information:

  - α where necessary to compute the constraint curve

  - Assumptions made and their rationale

- Plot the constraint diagram

  - Note down which constraints are active

  - Mark the feasible design space as well as your selected combination of TSLWTO and WTOS on the plot