AOE 4065: Air Vehicle Design

Homework 1: Mission Analysis

Due on or before 11 September 2025 (11:59 PM)

Instructions

- Your submission should be clear, concise, and complete. In preparing it, organize it logically in such a way that it is easy to comprehend and it contains sufficient detail within the specified constraints.

- Clearly state all relevant assumptions, and cite any reference material you use.

- Submit the homework to Canvas. Please use the following convention for naming the file: AOE4065_HW1_Last-name_First-name

Concepts

- Initial sizing to respond to an RFP

Purpose

To assess your ability to estimate initial take off gross weight (TOGW) of an aircraft concept.

Scenario:

A fighter jet is to be designed.

Aerodynamic characteristics are given to you:

ARwing=3

ewing=0.91443

From constraint analysis:

| WTOS=104.59 psf |  |
| --- | --- |

Payload:

- Fixed payload weight = 700 lbs

- Expendable payload weight = 4400 lbs

Mission:

|  | Takeoff | Climb | Cruise | Dash | Combat | Cruise | Loiter | Landing |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude, ft | 0 | 40000 | 40000 | 40000 | 25000 | 40000 | 10000 | 0 |
| Mach | 0.00 | 0.87 | 0.87 | 1.50 | 0.87 | 0.87 | 0.30 |  |
| Time, min |  |  |  |  | 2 |  | 20 |  |
| Distance, nm |  |  | 190 | 50 |  | 250 |  |  |
| Drop Payload, lb |  |  |  |  | 4400 |  |  |  |
| CD0 |  |  | 0.026996 | 0.026996 |  | 0.016996 | 0.016996 |  |
| TSFC, C, /s |  |  | 0.00019 | 0.00061 |  | 0.00019 | 0.00019 |  |

Deliverables:

One PDF file with the following information:

- For each mission segment, report the following information:

  - Weight fraction

  - Fuel used

  - Weight at the end of the segment

  - L/D where necessary to compute the weight fraction

  - Assumptions made and their rationale

- Report the converged TOGW

- Sensitivity studies. Plot TOGW vs. parameter

  - Fixed payload weight varies ±25%

  - TSFC in combat varies ±25%

Hint 1: Aerodynamics

The life-over-drag can be computed as

LD=1qCD0W/S+W/SqπeAR

Remember that WTO/S is given. The above equation uses W/S, the wing loading for the specific segment for which you are calculating L/D. Use the weight fraction that is being iterated to calculate W/S  during mission analysis. Thus, the wing loading, L/D, and weight fractions are being iterated on.

Hint 2: Propulsion

In combat segment, instead of the equation below:

WF=1-CTcombatWcombat t

You can use,

Fuel burn, in lb, during combat, Wfuel, combat=t×cT×TA

where,

Time of combat, t in minutes

TSFC, c=1.708 1hr

Thrust available, TA=9906.98 [lb]